MGENIE

Functionality of GENIE in Matlab

T.G.Perring

19 December 2005

Rutherford Appleton Laboratory Chilton DIDCOT Oxfordshire OX11 0QX

# MGENIE

Introduction and installation.............................................................................	 2

1. Reading ISIS data  ........................................................................................	 3

1.1  Assigning the ISIS location  ................................................................	 3

1.2  Reading neutron counts  ......................................................................	 4

1.3  Reading other data from the ISIS data source  ....................................	 8

2. Reading and writing ASCII files  ...............................................................	 9

2.1  Reading ASCII files  ..........................................................................	 9

2.2  Writing ASCII output files  .................................................................	11

3. Algebraic manipulation of Mgenie spectrum objects   ...............................	12

3.1  Arithmetic operations…........................................................................	12

3.2  Plotting Mgenie spectrum objects ........................................................	12

3.3  Plotting arrays of spectra (intensity maps) ….......................................	15

3.4  Spectrum manipulation commands  ………………………..…...........	15

4. Creating spectrum objects or accessing components of a spectrum   ......	20

4.1  Creating and altering a spectrum…......................................................	20

4.2  Creating and altering a tofspsectrum ...................................................	21

5. Interacting with MFIT   ..............................................           .....................	24

Appendix A: Valid raw file fields  ...................................................................	25

Appendix B: Troubleshooting  ........................................................................	29

Appendix C: MAPS alignment utilities   ………………………………........	30

C.1  Utility functions  ................................................................................	30

C.2  Use in crystal alignment  ....................................................................	31

Introduction and installation

MGENIE is a collection of routines that aim to provide the functionality of Genie within Matlab. No guarantee can be made that the functionality of the routines will not be altered, and the routines are certainly being regularly improved, extended and added to. Contact t.g.perring@rl.ac.uk to offer suggestions for improvements.

To try out MGENIE, do the following:

- installation:

- Install OpenGenie on your PC, if it is not already installed.

  - Go to the  OpenGenie homepage http://www.isis.rl.ac.uk/OpenGENIE/ and click on ‘Download’ in the bar across the top of the page. You must have OpenGenie installed because its data access routines are needed.

  - Fix the bug responsible for slow access to the DAE and CRPT by following the instructions at http://bugs.isis.rl.ac.uk/long_list.cgi?buglist=111

- Install MGENIE:

Go to the excitations group master file store (map a network drive to \\britannic\excitations$) go to \mprogs and copy the mgenie folder and all its sub-folders to your computer.

- In your Matlab startup.m (normally found in be found in c:\matlab6p5\work) add the paths to folders containing the mgenie code, and (optionally) an .m file to be run when mgenie is initialised (see below) :

path('...\mgenie\fortran\',path);

path('...\mgenie\',path);

mgenie_setup_file ('g:\matlab\mtoby\mgenie_setup.m');

where ‘...’ represents the rest of the path on your computer to the mgenie routines.

- using the routines in Matlab:

To initialise, type:

>> mgenie

This can also be put in your startup.m

- The initialisation file:

The .m file to be run when mgenie is initialised can be used to issue mgenie commands, for example, to set default locations in which to find raw data, default primary flight-path and monitor integral ranges for normalisation of data in high-level utilities (see Appendix C for examples of such utilities). An example is:

% set default locations for data and instrument

set_disk g:

set_dir /f90/

set_inst map

set_ext raw

% set primary flight-path for MAPS (leaves emode=0 i.e. diffraction)

set_primary(12.0);

% set monitor for normalisation, integration range, and monitor counts

% corresponding to unit normalisation:

analysis_init(1,1000,2000,1.0e6);

1. Reading ISIS data

1.1 Assigning the ISIS data location

 Assigning the location of .raw or .sav files:

Example: To read from MAPS raw file MAP03975.RAW located in G:\ISISDATA\MAPS :

>> set_disk (‘g:’)

>> set_dir (‘isisdata/maps’) 	! Note forward slash (‘/’) or backslash (‘\’) valid

>> set_inst (‘map’)			! The 3 letter abbreviation of instrument name

>> set_ext (‘raw’)			! ‘RAW’ or ‘SAV’

>> ass (3975)

Note:

- The instrument name is the three-letter abbreviation e.g. MAP, PRS, CRP …

- The full path is constructed from the arguments of SET_DISK and SET_DIR. How they are split is up to you. For example, you could put the disk to the blank string ‘’, and directory to the full path ‘g:/isisdata/maps’. Slashes are inserted as needed to separate the disk, directory and file names.

- User-defined path names can be set to create a search path recognised by mgenie. For example, to search g:/isisdata/maps, c:/temp and c:/my_data in that order:

>> set_globalpath(‘data_path’,‘g:/isisdata/maps’,‘c:/temp’,‘c:/my_data’);

>> set_disk (‘data_path’)

>> set_dir (‘’)

- On a PC running Windows, a forward slash ‘/’ can be used in placeof a backslash (‘\’). (It is a good habit to use the forward slash, not the conventional backward slash between directory names, for just as in many software packages, the backslash in Matlab is reserved for indicating special characters so sometimes it can cause problems).

To check the assigned run at any point, us SH_ASS:

>> sh_ass

 Assigning to the DAE or CRPT:

To read from the DAE or CRPT:

>> set_dae (‘maps’)		! Note the instrument name is given in full

>> ass (‘dae’)

>> set_crpt (‘maps’)

>> ass (‘crpt’)

Note:

- Here, the instrument name must be given in full e.g. MAPS, PRISMA, CRISP.

- It can sometimes take a long time for the first read of information from the DAE or CRPT, even with the OpenGenie fix explained in the installation. Be patient!

- A peculiarity of OpenGenie is that once the instrument for the DAE or CRPT has been chosen, they cannot be changed in that session. This means that you will have to exit Matlab to change the DAE or CRPT.

 Setting the moderator-sample distance and energy mode for units conversion:

The instrument (or energy)  mode, emode, (0,1,2 for elastic, direct geometry or indirect geometry), the fixed energy value, efix, and the moderator-to-sample distance, x2, need to be set before reading in data if units conversion is to be performed (see units command later on).

To set the energy and instrument mode:

>> set_efix (efix)		  % efix=0 elastic; efix > 0 direct; efix < 0 indirect geometry(meV).

and to set the moderator-sample distance:

>> set_primary (x1)	 % moderator-sample distance in metres.

To inquire of the energy, instrument mode and moderator-sample distance:

>> e  = get_efix

>> x1 = get_primary

1.2 Reading neutron counts

The following functions can be used to get counts from a raw file, sav files, CRPT or the DAE:

- spec		read one or more spectra and sum them into a single output spectrum

- mspec		read a list of spectra and map them into an array of output spectra

- mon		read a given monitor spectrum

- sumspec	create a list of integrals for a range of spectra

- gget		read raw counts

These are described in detail below:

 SPEC: read in a given spectrum number, or a range of spectra summed into a single output:

 Read a single spectrum, or sum a range of spectra into one spectrum:

e.g.	>> a = spec(34)			! reads 34th spectrum into a spectrum named ‘a’

>> asum = spec(257,512)		! sums spectra 257 to 512 into one spectrum

 The arguments can be vectors:

>> asum = spec([1,13,14,23])	! sums spectra 1,13,14, & 23 into one spectrum

>> asum = spec([1,101,201],[20,120,220])	! sum 1-20, 101-120, 201-220

 The spectra can also be taken from a mapping file:

>> asum = spec(map)		! sums all the spectra in the map file into one spectrum

The map file can be read from a file using the function load_map:

>> w1 = load_map (‘g:/f90/rings_943.map)	! reads from named file

>> w1 = load_map				 	! GUI file selector opened

(not read_map, which reads in the map fileas a spectrum suitable for graphical display)

The argument map is a cell array of one-dimensional arrays. You can construct one using curly brackets, for example:

>> map{1}=[1,3,4,6,7,8,144,200];	! cell array length one

>> map{2}=[1001,2000,3000];		! add a second element to the cell array

Notes:

- The output is counts per microsecond, with the x-axis being microseconds

- The object that holds the spectrum can be any valid Matlab variable name. This can include the familiar names W1, W2, W3 …

Options for controlling how the detector parameters are calculated, providing a mask file, and giving a period number are the same as described for mspec below.

 MSPEC: read in a range of spectra to give an array of output spectra:

 An array of spectra can be created with mspec:

e.g.	>> a = mspec(10)		! reads spectra just one spectrum – equivalent to spec(10)

>> a = mspec(1,512)	! reads spectra 1 to 512 into an array of 512 spectra

>> a = mspec(1,1024,16)	! reads spectra 1 to 1024 in groups of 16

In this example, a is an array: a(1) contains the sum of spectra 1 to 16, a(2) contains spectra 16-32 … a(64) contains spectra 1009 to 1024. To plot the 31st member of the array (which is the sum of spectra 497 to 512):

>> dh(a(31))	! The dh command stands for ‘display histogram’ – see below

 The arguments can also be vectors:

>> a = mspec([1,13,14,23])   ! array of spectra, 1st contains spectrum 1, 2nd spectrum 13 etc.

>> a = mspec([1,101,201],[20,120,220])

array of 60 spectra, in order spectra 1, 2,..20, 101, 102…120, 201, 202…220

>> a = mspec([1,10001,20001],[1000,11000,21000],250)

array of 12 spectra, in order spectra 1-250, 250-500, 750-1000, 10001-10250, 10251-10500,…, 20751-21000.

>> a = mspec([1,10001,20001],[1000,11000,21000],[250,500,1000])

array of 7 spectra, in order spectra 1-250, 250-500, 750-1000, 10001-10500, 10501-11000, 20001-21000.

 sum spectra according to a map file:

>> asum = spec(map)		! array of output spectra, one for each workspace of the

! map file.

The map file is the same as that used with spec: the map file can be read from a file using the function load_map:

>> w1 = load_map (‘g:/f90/rings_943.map)	! reads from named file

>> w1 = load_map				 	! GUI file selector opened

(not read_map, which reads in the map fileas a spectrum suitable for graphical display)

The argument map is a cell array of one-dimensional arrays. You can construct one using curly brackets, for example:

>> map{1}=[1,3,4,6,7,8,144,200];	! cell array length one

>> map{2}=[1001,2000,3000];		! add a second element to the cell array

Detector parameters: the detector parameters needed for units conversion (electronic delay time, sample-detector distance and scattering angle) are read from the assigned data source (raw file, CRPT or DAE). However, remember to set emode, efix and the moderator-sample distance before the call to spec or mspec with the functions set_efix and set_primary (see above).

The detector parameters returned by spec or mspec can be controlled by an optional final parameter average_mode:

>> a = spec (ilo, ihi, average_mode)

>> a = mspec (ilo, ihi, step, average_mode)

where:

average_mode 	= ‘average’		Average over all contributing detectors [default method].

= ‘min_twotheta’	Parameters taken from detector with minimum scattering angle.

= ‘max_twotheta’	Parameters taken from detector with maximum scattering angle.

= ‘none’	Zeros returned for detector parameters.

e.g. when using MAPS for crystal alignment and with detector to spectrum mapping of spec_mid-tubes_022.dat, then to get the scattering angles corresponding to the pixels in the horizontal plane:

>> w = mspec(1,576,’min_twotheta’)

If speed of access is critical then average_mode=’none’ can be chosen. However, units conversion will not then be possible.

Mask files: a list of spectra to be removed from the summation performed by spec or mspec can be supplied. It is a one dimensional array containing the list of spectra to be masked.

>> msk_array = [32,33,47,412];

>> w = mspec(1,576,’mask’,msk_array)

Mask arrays can be read from a file, for example on the chopper instruments created using DIAG, using the function load_mask described elsewhere. (NOT read_mask, which reads from the same file into a spectrum suitable for plotting.)

Period number: if a mapping of raw file spectra contains spectra only from the first period, then the same mapping can be used to get data from the corresponding spectra in a later period by giving the period number:

>> w = mspec(1,576,’period’,5)	% get counts for 5th period.

The functions spec and mspec offset the spectrum numbers accordingly to get the data from the requested period.

 MON: read in the spectrum for a given monitor number:

e.g.	>> white_mon = mon(1)		! reads monitor 1

>> m2 = mon(2)			! reads monitor 2

The period number and average_mode=’none’ can optionally be specified:

e.g.	>> white_mon = mon(1,17)	! reads monitor 1 for period 17

>> m2 = mon(2,’none’)		! reads monitor 2 with zeros returned for det. parameters

>> m3 = mon(3,17,’none’)	! reads monitor 3 with zeros returned for det. parameters

 SUMSPEC: obtain a workspace of spectrum integrals:

To obtain a workspace that contains the integrals for each spectrum over all time of flight:

>> w_out = sumspec (sp_lo, sp_hi)

For the integrals over a the time range T_LO to T_HI:

>> w_out=sumspec(sp_lo, sp_hi, t_lo, t_hi)

e.g.	>> wsum = sumspec(1,41472)	! integrals for all MAPS spectra

>> wsum = sumspec(101,356,1000,5000)

Note:

- Use this command rather than successively calling SPEC and then integrating the output using the INTEGRATE command, as SUMSPEC is much more efficient.

- SUMSPEC can be used on arbitrarily large ISIS raw files. (For those who care about these matters: sumspec limits the memory consumption in intermediate calculations by handling only 512 spectra at a time.)

 GGET(‘CNT’): read raw counts:

To obtain counts a function of time channel number for a range of spectra into a 2D array:

e.g.	>> counts = gget(‘cnt1’)		! reads counts for all spectra, i.e. 0 to NSP1

>> counts = gget(‘cnt1[15:35]’)	! reads counts for spectra 15 to 35

In both examples, a simple two dimensional array is created, which contains raw counts as a function of time channel index. Normally it is more appropriate to use SPEC to read data, but where the conversion to counts per microsecond is not required, or a large block of data is wanted, this command reads the data much more rapidly than a loop containing calls to SPEC.

In the first example: COUNTS has dimensions COUNTS (NTC1+1, NSP1+1)

where 	NTC1 = no. time channels

NSP1 = maximum spectrum number

Note that the in the output array, the first time channel for each spectrum corresponds to a garbage collection time channel: it will contain counts for that spectrum that the data acquisition electronics cannot place in the other time channels (it is the time axis equivalent of spectrum zero).

Thus:

>> counts = gget(‘cnt1’)

>> c = counts(2:end, 1)		! counts in spectrum zero

>> c = counts(2:end, 2)		! counts in spectrum 1

:

>> c = counts(2:end, ntc1+1)	! counts in spectrum NCT1

In the second example,

>> counts = gget(‘cnt1[15:35]’)

>> c = counts(2:end, 1)		! counts in spectrum 15

>> c = counts(2:end, 2)		! counts in spectrum 16

:

>> c = counts(2:end, 21)		! counts in spectrum 35

It is necessary to convert the output using CNT1 to a floating point array in order to perform any algebra with the output, just as for any integer in Matlab. For example, to get a section of the raw counts and their associated error bars:

e.g.	>> counts = double(gget(‘cnt1[15:35]’))

>> err = sqrt(counts)

1.3 Reading other data from the ISIS data source

Other information in the raw file, such as run title, microamp-hours and the detector tables, can be obtained with the function gget:

>> {output_object_name} = gget (‘{object_name}’)

The object is returned can be an integer, real or character variable, and is a scalar or an array according as the description in the definition of the raw file format:

e.g.	>> ndet = gget(‘ndet’)		! No. of detectors

>> spec_array = gget(‘spec’)	! Array of spectrum numbers corresponding to each detector

! Thus SPEC_ARRAY is an array length NDET.

>> run_title = gget(‘titl’)	! Run title (character string)

>> user = gget(‘user’)		! User information (array of character strings)

In addition, GGET recognises a number of other names in order to extract useful information from their locations in obscurely named array fields within the raw file (see ‘For afficionados’ below). A full list of names recognised by GGET is given in Appendix A. Examples include:

>> uamp = gget(‘good_charge’)		! microamp.hours returned as a floating number

>> nframe = gget(‘good_frames’)	! number of good frames returned as an integer

>> date = gget(‘end_date’)		! end date returned as a character array

Note: in Matlab, to do any algebra with integers they must be converted into floating numbers first. For example, to plot a monitor spectrum normalised by the number of good frames:

>> m1_norm = mon(1)/double(gget(‘good_frames’))

>> dh(m1_norm)

For afficionados:

Some information is stored in the raw file deep in obscurely named arrays, whose elements have to be interpreted variously as integer, real or character according to the information they contain. For example, in the raw file fields definition, there is an array RPB, of length 32 elements, in which RPB(8) is the number of uA.hr and should be read as a real; RPB(11) is the number of good frames, to be read as an integer; and RPB(17) is the finish date, a character string. In such mixed type objects, prefix the object name with R, I or C according to whether an integer, real or character interpretation is required, and pick out the required element:

e.g. 	>> real_rpb = gget (‘rrpb’)	! real_rpb(8) is uA.hr

>> real_rpb(8)

ans =

5.3388e+3

>> int_rpb = gget (‘irpb’)		! int_rpb(11) is good frames

>> ch_rpb = gget (‘crpb’)		! ch_rpb(17) is finish date

>> ch_rpb(17)

ans =

‘ 4-AUG-2002’

It is the more commonly required elements from arrays like RPB that are given names that are recognised by GGET.

2. Reading and writing ASCII files

# 2.1 Reading ASCII files

 Point data (columns of X-Y or X-Y-E data)

e.g.	>> w1 = read_points (‘c:/temp/test.dat’) 		% reads from named file

>> w1 = read_points			 		% GUI file selector opened

Format of point data file:

x(1)	y(1)	e(1) 				x(1)	y(1)

x(2)	y(2)	e(2) 		or		x(2)	y(2)

:	 :	 :				 :	 :

x(n)	y(n) 	e(n) 				x(n)	y(n)

This is identical to the function  read_xye:

e.g.	>> w1 = read_xye (‘c:/temp/test.dat’) 		% reads from named file

>> w1 = read_xye			 			% GUI file selector opened

 Histogram data (columns of Bin boundary-Y or Bin boundary-Y-E data)

e.g.	>> w1 = read_hist (‘c:/temp/test.dat’) 		% reads from named file

>> w1 = read_hist			 			% GUI file selector opened

Format of histogram data file: bin boundaries must be given, y(n+1) and e(n+1) are dummy values

x(1)	y(1)	e(1) 				x(1)	y(1)

x(2)	y(2)	e(2) 		or		x(2)	y(2)

:	 :	 :				 :	 :

x(n)	y(n) 	e(n) 				x(n)	y(n)

x(n+1)	0.0	0.0				x(n+1)	0.0

 General ASCII file reading

e.g.	>> w1 = read_ascii (‘c:/temp/test.dat’) 		% reads from named file

>> w1 = read_ascii			 		% GUI file selector opened

Attempts to read x-y or x-y-e data from an ASCII file, skipping over leading lines of text, and determining if point or histogram data from the number of entries on the last line. See >> help read_ascii for more details.

 SUM file as produced by Homer, Sum, White_van, SUMSPEC

e.g.	>> w1 = read_sum (‘g:/f90/map04291.sum’)		% reads from named file

>> w1 = read_sum				 		% GUI file selector opened

The result is a point-mode spectrum.

 MASK file as produced by DIAG

e.g.	>> w1 = read_mask (‘g:/f90/map04291.sum’)		% reads from named file

>> w1 = read_mask				 		% GUI file selector opened

The result is a point-mode spectrum where masked spectra are given the value 1 and unmasked spectra are given the value 0.

Note that because the total number of spectra in the raw file is not recorded in the mask file, the x axis runs from 1 to the highest mask spectrum number. To use the mask file when manipulating, for example, sum file data read with READ_SUM, ensure that the number of bins in the mask spectrum matches that of the sum file by using the rebin command:

>> wsum = read_sum(‘het11179.sum’)

>> wmsk = read_msk(‘het11179.msk’)

>> rebin(wmsk,wsum)

 Cut file as produced by Mslice

e.g.	>> w1 = read_cut (‘g:/f90/map04291.sum’)		% reads from named file

>> w1 = read_cut				 		% GUI file selector opened

This command will read the x,y,e values for the cut into a point-mode spectrum.

 Slice file as produced by Mslice

e.g.	>> w1 = read_slice (‘g:/f90/map04291.sum’)	% reads from named file

>> w1 = read_slice				 	% GUI file selector opened

This command will read the x,y,e values for the cut into an array of class spectrum. To plot, for example, the 7th row of the slice file,

>> wslice = read_slice;

>> dh(wslice(7))

or:

>> mplot (wslice)		% for a 2D plot

 SPE file as produced by Homer (the name read_spefile is chosen to avoid conflict with Mslice)

e.g.	>> w1 = read_spefile (‘g:/f90/map04291.spe’)	% reads from named file

>> w1 = read_spefile				 	% GUI file selector opened

This command will read the .SPE file into an array of class spectrum. To plot, for example, the 7th workspace in the .SPE file,

>> wspe = read_spefile;

>> dh(wspe(7))

or:

>> mplot (wspe)		% for a 2D plot

*** Do not confuse with the similar mslice utility read_spe, which has a different syntax and output.

*** Do not try to read in a MAPS SPE file – it will be too slow.

 Text file

e.g.	>> w1 = read_text (‘g:/experiments/my_stuff.txt)	% reads from named file

>> w1 = read_text				 		  	% GUI file selector opened

Reads lines of text from an ASCII file into a cell array. Currently limited to a maximum of 10,000 lines.

 Text file read-and-write

e.g.	>> read_write (nlines, file1, file2)

>> read_write (nlines)		 	% GUI file selector opened

>> read_write			 	% GUI file selector opened, upto 1000 lines copied

Reads up to nlines lines of text from an ASCII file and writes to the output file. Currently limited to a maximum of 1,000 lines. Some test files are too large to be opened in standard text editors. This function provides a convenient way of copying the first part of a file to be edited in a standard text editor.

2.2 Writing ASCII output files

# Writing a histogram spectrum to a file

e.g.	>> write_hist(w1,’c:/temp/my_output.dat’)

Creates a data file with format:

x(1)	y(1)	e(1)

x(2)	y(2)	e(2)

:	 :	 :

x(n)	y(n) 	e(n)

x(n+1)	0.0	0.0

# Writing x-y-e data to a file

e.g.	>> write_points(w1,’c:/temp/my_output.dat’)

or equivalently:

e.g.	>> write_xye(w1,’c:/temp/my_output.dat’)

Creates a data file with format:

x(1)	y(1)	e(1)

x(2)	y(2)	e(2)

:	 :	 :

x(n)	y(n) 	e(n)

3. Algebraic manipulation of Mgenie spectrum objects

MGENIE treats spectra as an object whose structure is hidden from the user. It is this that enables functions to be written that define addition, multiplication etc. on the spectra. There are two types of spectrum available in MGENIE: the basic ‘spectrum’ and a ‘tofspectrum’. The basic spectrum holds x,y,e arrays, captions, and is used, for example, when data is read in from an ASCII file. Data read from a raw file contains in addition the distance, angles and energies needed for units conversion. All the algebraic operations, functions and plotting routines apply equally to both types of spectrum. For details on manipulating the individual fields within a spectrum, see Section 4.

3.1 Arithmetic operations

The usual arithmetic operations are available with the usual Matlab syntax. They work for algebra of spectra and scalars.

Addition	+

Subtraction	-

Division	/

Multiplication	*

Raise to power	^

e.g.	>> m1 = mon(1);

>> m2 = mon(2);

>> m3 = mon(3);

>> w_out = (m1+m2^0.6 /(m3+m1/100000)

The algebraic operations also apply to arrays of one or both of the arguments:

e.g.	>> w=mspec(1,4); 	% w is a tofspectrum array of length 4, the first element of the array

% containing spectrum 1, the second containing spectrum 2 etc. (see mspec)

>> w=w+100;		% add 100 to the y axis for each of the 4 spectra in the array

>> w=w+[1,2,3,4]	% add 1 to the first spectrum, 2 to the second etc.

3.2 Plotting MGENIE spectrum objects

To create new plots:

>> dl (w1)	! Create new plot of W1 drawn as a line joining points/bin centres

>> dm (w1)	! marks at points/bin centres

>> de (w1)	! error bars at points/bin centres

>> dh (w1)	! histogram (if point data, the boundaries are drawn at half-way between the points)

The general format of the display commands DL, DM, DE and DH allows the limits to be set:

>> dl(w1)				! Draw line with limits with auto-scaling on x and y axes

>> dl(w1,xlo,xhi,ylo,yhi)	! Draw line between xlo and xhi, with the y axis auto-scaled

>> dl(w1,xlo,xhi,ylo,yhi)	! Draw line with defined limits on both x and y axes.

The function dl, dm etc are used to plot a single spectrum. If warr is an array of spectra, then to plot one element of that array refer to it by its index e.g.

>> dl(warr(4))

[Intensity maps of spectrum arrays can be created using the mplot command described later.]

Overplot on an existing graph:

>> pl (w1)	! overplots a line joining the points/bin centres on an existing plot

>> pm (w1)	! likewise marks

>> pe (w1)	! likewise error bars

>> ph (w1)	! likewise histogram

New limits cannot be set with the overplotting commands PL, PM, PE and PH.

Altering binning for graphics output:

>> bin (n)	! rebins graphics in groups of N bins

e.g. to draw a line, and then overplot the histogram for the same data with points binned in groups of 4:

>> bin(1)

>> dl(w1)

>> bin(4)

>> ph(w1)

This command is the equivalent of the Genie-II ALTER BIN N command. The workspace is unaltered by BIN; it only controls display. To rebin the workspace itself, use the REBIN, REGROUP or REBUNCH commands described below.

Changing limits:

On an existing graph:

>> lx (xlo,xhi)

>> ly (ylo,yhi)

>> lx	% changes the x-axis range to include the full data range of the plotted spectra

>> ly	% changes the y-axis range to include the full data range for the presently plotted x-axis range

Linear or logarithmic axes:

To change the axes to logarithmic for the existing graph and change the default for future plots:

>> logx

>> logy

or change to linear axes:

>> linx

>> liny

Altering linestyle, linewidth, marker type, marker size and colour:

 To set the colour to be used for lines and markers:

>> acolor (color)

where color is the standard Matlab character:

‘r’, ‘g’, ‘b’, ‘c’, ‘m’, ‘y’, ‘k’, ‘w’

or an unambiguous abbreviation of the full colour name:

‘red’, ‘green’, ‘blue’, ‘cyan’, ‘magenta’, ‘yellow’, ‘black’, ‘white’

A series of colours can be set for use in plotting commands that cascade several plots. The settings will be cycled through

e.g.   	>> acolor('b’,’r’,’bla’,’g’)		! plotted lines will cycle blue, red, black,green …

 To alter line properties:

>> aline (line_width)				! line_width only

>> aline (line_style)				! line_style only

>> aline (line_width, line_style)		! both (order does not matter)

Valid line styles: type either the Matlab code or text equivalent (or minimum abbreviations)

'-'  		‘solid’

'--' 		‘dashed’

':'   	‘dotted’

'-.'  	‘ddot’ 	(dashed-dot)

The line width is given in point size; the default size is 0.5

e.g.   	>> aline('-')

>> aline('da',10)

>> aline(2)

>> aline(0.5,':')

A series of line widths and/or styles can be set for use in plotting commands that cascade several plots. The settings will be cycled through

e.g.   	>> aline('-',’--’,’:’)		! plotted lines will cycle -, --, :, -, --, :, …

>> aline(0.5,1)			! plotted lines will cycle widths 0.5,1,0.5,1,…

 To alter marker properties:

>> amark (marker_size)			! marker size only

>> amark (marker)				! marker_type only

>> amark (marker, marker_size)	! both (order does not matter)

Valid marker types are the standard Matlab markers:

'+', 'o', '*', '.', 'x', 's', 'd', '^', 'v', '>', '<', 'p', 'h'

where	'd'   			diamond

'p', 'h'   		5 or 6 pointed stars (pentagram, hexagram)

'^', 'v', '>', '<' triangles

The size of markers is given in points (default size is 6)

e.g.   	>>  amark('+')

>>  amark('+',10)

>>  amark(6)

>>  amark(10,'+')

A series of marker sizes and/or styles can be set for use in plotting commands that cascade several plots. The settings will be cycled through

e.g.   	>> amark('x',’+’,’*’)		! plotted marks will cycle x, +, *, x, +, *, …

>> aline(4,6)			! plotted marks will cycle sizes 4,6,4,6,…

Other graphical features:

 Write text on the graphics screen:

>> cursor		% for horizontal text

>> cursor v		% for vertical text

Cross-hairs appear on screen. Left-click mouse button and use the pop-up menu to choose to mark the position of the cursor or write text at the position of the cursor.

 Return the x-y position of the cursor position on the graphics screen:

>> xy

Cross-hairs appear on the screen. Left-click the mouse to give the (x,y) position of the cross-hair. Result printed to screen

3.3 Plotting arrays of spectra (intensity maps)

Use the mplot function to plot an array of spectra:

>> mplot (w)

By default, the y-axis is the spectrum number i.e. the y-values are 1,2,3…. Different y-axis values can be supplied by further arguments:

e.g.: the array of spectra, w, might be a series of cuts taken at different temperatures. In this case, fill an array yval with a list of temperatures (column or row), and type:

>> mplot (w, temperature)

Full Syntax:

>> mplot (w)    	% y axis values are spectrum number i.e. 1,2,3 ...length(w)

The bin boundaries along the y-axis are 0.5, 1.5, 2.5, ... length(w)+0.5

However, if there is control information in the title, then this will be used if possible

>> mplot (w, yval)   % y values are given by the array yval.

Ensure yval is monotonically increasing.

If length(yval) = length(w):

bin boundaries are given by the half-way points between the values of yval.

if length(yval) = length(w)+1:

yval contains the bin boundaries

>> mplot (w, ystart, ystep) % y values are ystart, ystart+ystep, ystart+2*ystep, ystart+3*ystep ...

Bin boundaries are ystart-step/2, ystart+step/2, ystart+(3/2)*ystep ...

>> mplot (w, yval, ywidth)  % y values are in the array yval, bin-widths in the array ywidth

To alter the plot limits, and linear/logarithmic scales, use the following commands, with syntax just as for plotting single spectra:

>> lx		>> ly		>> lz

>> linx		>> logx

>> liny		>> logy

3.4 Spectrum manipulation commands

# The following commands manipulate a single spectrum or time-of-flight spectrum, or arrays of time-flight spectra. For example, to convert all the data from a MAPS white beam alignment run to d-spacing:

:

>> ass(5705)

>> wt = mspec (1,576,’min’);	% Read in data from a run with the mid-tubes spectra.dat

% one instrument spectrum per element of wt

>> wd = units(wt,’d’);		% Converts to an array of 576 spectra in d-spacing

Where numerical arguments need to be provided in addition to the input spectrum or array of spectra, then these must be scalar. For example in the shift function which translates the spectra along the x-axis:

>> warr = mspec(1,3)		% create an tofspectrum array of length 3

VALID	>> wout = shift(warr,50)	% shift all three tofspectra along the x-axis by 50 s

INVALID	>> wout = shift(warr,[10,20,30])	% attempt to shift 1st spectrum, by 10 s, 2nd by

% 20 s, 3rd by 30 s

# DERIV

Numerical derivative of a spectrum.

Format:

>> wout = deriv(w_in)

# INTEGRATE

Integrate a spectrum between two limits. The output is a structure that has tow components: ans.val = integral,  ans.err = error on the integral.

Format:

>> ans = integrate(w_in,xlo,xhi)

If w_in is an array of spectra, then the result is a spectrum containing the integrals for each of the spectra in the array e.g.

>> w_in = mspec(1,1024,16)		% Create an array of 64 spectra which contain spectra

1-1024 in groups of 16

>> sum = integrate(w_in,1000,2000)	% A spectrum with x axis =1,2,…64 containing the

integrals from 1000 to 2000 s

See also SUM_WORK below.

# REBIN

Rebins data with new bin boundaries.

Formats:

>> w_out = rebin(w_in,xlo,xhi)

Keeps only data between xlo and xhi, maintaining original bin boundaries between

>> w_out = rebin(w_in,xlo,dx,xhi)

Keep only data between xlo and xhi, with bin boundaries in between defined by:

dx > 0	constant bin width = dx

dx < 0 	logarithmic binning x(m+1) = x(m)(1+|dx|)

dx = 0	retain original bin boundaries

>> w_out = rebin(w_in,[x1,dx1,x2,dx2,x3...,xn,dxn,xn+1])

Keep only data between x1 and xn+1, with bin boundaries in the various intervals between defined by:

dxi > 0	constant bin width = dxi

dxi < 0 	logarithmic binning x(m+1) = x(m)(1+|dxi|)

dxi = 0	retain original bin boundaries

The binning parameters can be put in an array

e.g. >> xbounds = [1000,10,1500,0,2000,-0.01,3000]

>> wout = rebin(w1,xbounds)

>> w_out = rebin(w_in,w_ref)

Rebins the data of w_in with the bin boundaries of w_ref. (Note this syntax is reversed from Genie-2)

# REBUNCH

Rebins the data by adding together NBIN successive bins. It is equivalent to the Genie-2 command ALTER BINNING, except that the output is a regrouped spectrum, rather than an action simply to be performed for graphical output.

Format:

# >> w_out = rebunch(w_in, nbin)

# REGROUP

Regroups data with new bin boundaries to ensure that bins have minimum width determined by the parameter dx, but ensuring the bin boundaries are always coincident with original bin boundaries.

Format:

>> w_out = regroup(w_in,xlo,dx,xhi)

Keep only data in bins entirely contained in the range xlo to xhi, with bin boundaries in between having minimum width determined by:

dx > 0	constant bin width = dx

dx < 0 	logarithmic binning x(m+1) = x(m)(1+|dx|)

dx = 0	retain original bin boundaries

For point data only, the regroup function is identical to rebin, and takes the full syntax of rebin.

The difference between REBIN and REGROUP is that in the former the data in the original bins may be divided by the new bin boundaries. In the latter case, new bins are created only by combining whole bins. This is true also for the ends of the regrouped array: if the bin boundaries are 990,1010,1030,1050,…,1210, then REGROUP(w1,1000,25,1200) yields a workspace with bin boundaries 1010,1050,1090,1130,1170.

# SHIFT

Shifts the bin boundaries of a spectrum. In the example below, if XSHIFT is positive, then the bin boundaries are increased uniformly by XSHIFT; if negative, then the boundaries are decreased uniformly.

Format:

>> w_out = shift(w_in, xshift)

# SUM_WORK

If w_in is an array of spectra, then sum the spectra between the lower and upper indices. The result is a single spectrum.

Format:

>> ans = sum_work(w_in,ilo,ihi)

e.g.

>> w = mspec(1,1024,16)		% Create an array of 64 spectra which contain spectra

1-1024 in groups of 16

>> wsum = sum_work(w, 10, 20)	% Create a single spectrum wsum = w(10)+w(11)…+w(20)

(See also INTEGRATE above.)

# UNSPIKE

Removes spikes from data, with optional parameters to refine the criteria for a spike. Points that are deemed to be spikes are reset to the linear interpolation of the bracketing good data.

Format:

Use default refinement criteria:

>> w_out = unspike(w_in)

Full format:

>> w_out = unspike(w_in, ymin, ymax, fac, sfac)

There are four optional parameters:

YMIN    	If y < ymin then the point is considered a spike [Default: NaN i.e. ignored].

YMAX    	If y > ymax then the point is considered a spike [Default: NaN i.e. ignored].

FAC     	If a point is within a factor FAC of both of its neighbours then is NOT a spike [Default: FAC = 2].

SFAC   	If the difference of a point w.r.t. both of its neighbours is less than SFAC standard deviations then the point is NOT a spike [Default: 5].

Use NaN to skip over an optional parameter , for example:

>> wout = unspike (w1, NaN, NaN, 1.5)   % to alter FAC to 1.5

# UNITS

Change the units of a time-of-flight spectrum.

>> w_out = units(w_in, units_type)

e.g.	>> wd = units (wt, ‘d’)		% d-spacing (valid if emode=0)

Valid units (depending on the energy mode) are:

t       	time-of-flight

v       	neutron velocity

tau     	inverse neutron velocity

lam    	wavelength

k       	wavevector

e       	neutron energy

d       	d-spacing

w       	energy transfer (meV)

wn     	energy transfer (cm^-1)

thz     	energy transfer (THz)

q       	momentum transfer

sq      	square of momentum transfer

4. Creating spectrum objects or accessing components of a spectrum

MGENIE treats spectra as an object whose structure is hidden from the user. It is this that enables functions to be written that define addition, multiplication etc. on the spectra. However, it can be useful to create a spectrum from separate arguments, or access the structure of a spectrum object on occasion.

There are two types of spectrum available in MGENIE: the basic ‘spectrum’ and a ‘tofspectrum’:

‘spectrum’:

Contains x,y,e arrays, caption information, character string for the x-axis units and whether or not the y axis is a distribution in the x axis (e.g. counts per microsecond as a function of time).

‘tofspectrum’:

Contains in addition, the time-of-flight parameter information (energy mode, delay time, fixed energy, primary and secondary flight-paths, scattering angle and fixed energy), and the current units of the x-axis (this is used to set the units character string in the ‘spectrum’ component).

When time-of-flight data is read from a raw file, it is read in as a tofspectrum. If data is read, for example, from a text file containing x-y-e data using read_hist or read_xye, or from an Mslice .cut file using read_cut, the data is placed as a simple spectrum, as the data may not be time-of-flight data.

4.1 Creating and altering a spectrum

4.1.1 Fields of a spectrum

x 	Array of x values

y        	Array of y values

e     	Array of standard errors

[	If length(x) = length(y)+1, then x values are taken as histogram bin boundaries.

If length(x) = length(y), then (x,y) are taken as points on a curve. ]

title     	Title for graphical display

xlab    	X-axis label (excluding any explicit units declarations - see below)

ylab    	Y-axis label (excluding any units arising from explicit declaration of being a

distribution - see below)

xunits 	Units of x-axis. e.g. 'meV'. The units will be incorporated into the x-axis label

displayed in any graphics output

distribution	=0 if y values are not a distribution in the units of the x-axis (default)

=1 if y values are a distribution

If distribution=1, then the inverse of units of the x-axis, as given by xunits, will

be used in the y-axis label displayed in any graphics output.

e.g. if xlab = 'Energy transfer', ylab='Normalised counts', xunits = 'meV', and distribution=1, then the labels displayed on the x and y axes on a plot will be 'Energy transfer (meV)' and 'Normalised counts / meV'

4.1.2 Creating a spectrum from arrays

>> w = spectrum(x,y)                                 			! x,y, arrays only

>> w = spectrum(x,y,e)                            			! x,y,e arrays only

>> w = spectrum(x,y,e,title,xlab,ylab)   		! with title and axis labels

>> w = spectrum(x,y,e,title,xlab,ylab,xunit)   	! with x-axis units

>> w = spectrum(x,y,e,title,xlab,ylab,xunit,dist)  ! y values a distribution in x-axis

! units if dist=1, (default:dist=0)

Special cases are:

>> w = spectrum(win)    		! win is a structure with all the fields of a spectrum

>> w = spectrum         		! create an empty spectrum

Notes:

  a spectrum can contain histogram or point data:

- to create a histogram spectrum:

x = row or column vector of bin boundaries: [x1, x2, x3,…, xn, xn+1]

y = row or column vector of intensity per unit measure of x: [y1, y2, y3,…, yn]

e = row or column vector of standard deviations per unit measure of x: [e1, e2, e3,…, en]

- to create a point spectrum:

x = row or column vector of x coordinates: [x1, x2, x3,…, xn]

y = row or column vector of y coordinates: [y1, y2, y3,…, yn]

e = row or column vector of standard deviations: [e1, e2, e3,…, en]

 Captions: title, xlab, ylab can be character arrays, or cell arrays of character strings

e.g.   xlab = {'hello' 'its' 'me'}     or   = {'hello' ; 'me old mate'}      or   = ['hello' ; 'tiger']

4.1.3 Altering the components of a spectrum

Fields can be replaced using the usual syntax of the Matlab set command:

>> win=set(win,‘field_name1’,new_value1,‘field_name2’,new_value2,...)

e.g.	>> new_y = [1,2,3,4,5];

>> str = ‘My new title’

>> win = set(win, ‘y’, new_y,’title’,’str’)

>> win = set(win,’ylabel’,’currant bun’)

4.1.3 Accessing and manipulating the components of a spectrum

>> w = get(w_in)

From the input spectrum W_IN this creates the output structure w that contains the all the field described above.

One way to manipulate components of a spectrum is illustrated by the following example:

>> w = get(win)

>> xcent = middle (w.x)		! get bin centres (only needed if w is a histogram spectrum)

>> w.y = w.y/xcent.^2		! correct for Q2 factor in phonon cross-section

>> w.e = w.e/xcent.^2

>> win = spectrum(w)

4.2 Creating and altering a tofspectrum

A ‘tofspectrum’ is a time-of-flight spectrum. It is made from

- a spectrum object (as described above)

- a time-of-flight parameter object (‘tofpar’)

- a string containing the units status for units conversion

4.2.1 Spectrum object

As described above.

4.2.2 Tofpar object

Fields of a tofpar:

emode		0,1,2 for elastic, direct geometry or indirect geometry

delta		Electronic delay time of the detectors (s)

x1		Moderator-sample distance

x2		Sample-detector distance

twotheta	Scattering angle (degrees)

efix	Fixed energy value (meV) : incident energy if direct geometry, final energy  if   indirect geometry.

 Creating a tofpar object:

>> par=tofpar(emode,delta,x1,x2,twotheta,efix);	% object with the given parameters % (all must be supplied)

Special cases:

>> par=tofpar(pstruct);		% pstruct is a structure with all the fields of a tofpar object

>> par=tofpar;			% blank tofpar

 Altering an existing tofpar object:

>> par=set(par, name_1, value_1, …)

e.g.

>> par=set(par,‘delta’,9.5,‘x2’,6.0,‘twotheta’,3.2626)

 Accessing the fields of a tofpar object:

>> pstruct=get(par)	% fill a structure with the fields of par

Just as for spectra, this gives an alternative way of changing the fields ro creating a new object based on an existing one:

>> pstruct=get(par)

:

<manipulate fields of pstruct>

:

>> par_new=tofpar(pstruct)	% reassemble a tofpar object

4.2.3 Units string

Used by units conversion command. Valid units are:

‘t’       	time-of-flight

‘d’       	d-spacing

‘v’       	neutron velocity

‘tau’     	inverse neutron velocity

‘lam’    	wavelength

‘k’       	wavevector

‘e’       	neutron energy

‘d’       	d-spacing

‘w’       energy transfer (meV)

‘wn’     energy transfer (cm^-1)

‘thz’     energy transfer (THz)

‘q’       	momentum transfer

‘sq’      square of momentum transfer

4.2.4.Creating and accessing the components of a tofspectrum

 Creating a tofspectrum from a spectrum:

>> w=tofspectrum(spec, par, units)	% Create from spectrum, tofpar objects and units

>> w=tofspectrum(struct)	% struct is a structure with the fields of a spectrum or of a tofspectrum

>> w=tofspectrum				% blank tofspectrum

 Altering an existing tofspectrum:

The parts of an existing tofspectrum can be altered as follows:

>> w = set_spectrum (w, spec)	% spec is an object of class spectrum; the existing % time-of-flight parameters & units are unchanged

>> w = set_par (w, par)			% par is an object of type tofpar

>> w = set_par(w,’emode’,2,’x1’,12)	% set fields of the time-of-flight parameters

>> w = set_units (w, ‘t’)

 Accessing the components of a tofspectrum:

>> wstruct = get(w)

Creates a structure containing all of the fields of a spectrum, tofpar and units in that order. The individual fields can be altered, and then the structure repackaged, just as the example of a spectrum:

>> ws = get(w);

:

<manipulate fields>

:

>> w = set(ws);

e.g.

>> ws = get(w);

>> ws.title=’New title’;

>> ws.efix=37.4;

>> ws.units=’d’		% dangerous: messing about with units can cause havoc!

>> w = tofspectrum(ws);

5. Interacting with MFIT

5.1 Passing a spectrum or a tofspectrum to Mfit*

Syntax*:

>> tomfit (win)

>> tomfit (win, xrange)

>> tomfit (win, xrange, p)

>> tomfit (win, xrange, p, fixed)

where:

win       	Mgenie spectrum to be fitted

xrange  	Array giving a values to be retained for fitting:

e.g. [12.3,14.5]            keep data in the range 12.3 =< x =< 14.5

e.g. [4.1,5.6;11.2,14.5]    keep 4.1=<x=<5.6 & 11.2=<x=<14.5

p       	Starting parameter values

fixed   	Array of length(p), teh fixed parameters (0: free, 1: fixed)

e.g. if 5 parameters, [0,1,1,0,1]  fix 2nd, 3rd, 5th.

If you want to pass parameter values and/or fixed values, but not limit the x range, then call as

>> tomfit (w, [], p, fixed)

or

>> tomfit (w, '', p, fixed)

5.2 Calculating y values using current parameters and function in Mfit*

To return a spectrum or tofspectrum with the y values calculated at the x-values of an input spectrum using the latest parameters and function in Mfit:

Syntax*:

>> wcalc = fromfit (w)                    		% most common use

>> [wcalc, p, dp, fixed] = fromfit (w)	% to recover parameter values etc

The state of Mfit (i.e. current x,y,e,selected range, parameter values etc.) is unaltered by the fromfit command.

* Note: in older versions of mgenie the function tomfit and fromfit were w_tomfit and w_fromfit. The older names are still recognised to allow backwards compatibility. However, it is recommended that the newer names are used.

Appendix A: Valid raw file fields

This list gives names that are recognised by GGET. They include all valid raw file field names (more details at http://ndadixwell/ICSG/Papers/P05-Raw%20File%20Fields.htm ), and additional names that are recognised by GGET. The more useful field names are highlighted.

In the following table, the return argument types are given by the notation:

- R		real number

- R(n)		real array of length n

- I		integer

- I(n)		integer array length n

- c*24		character string length 24

- c*(*)(5)	cell array of n strings,

Note:

- If the return argument is an integer or integer array, then to use it in algebraic manipulations convert the output to a floating point number:

e.g.   	>> norm_factor = 1/double(gget(‘good_charge’))

- If the return argument is a cell array of strings, then to convert an individual element of the cell array into a conventional character string, use the Matlab function CHAR. For example, to read the 4th string from the array user in the Run section:

>> c = gget(‘user’);

>> r = char(c(5))

Parameter names recognised by GGET:

	rid		c*8		Run identifier e.g. MAP03975

	short_title		c*24		Short title (identical to first 20 characters of TITL)

	start_date		c*11		Format dd-mm-yyyy

	start_time		c*8		Format hh:mm:ss

	end_date		c*11		Format dd-mm-yyyy

	end_time		c*8		Format hh:mm:ss

	run_duration		I		Length of time in units given by RUN_SCALER

	run_scaler		I		Unit of measure of RUN_DURATION: 0,1,2 = seconds, 3=frames

	good_charge		R		Good microamp.hours (for character equivalent, use UA_HOURS)

	total_charge		R		Total microamp.hours

	good_frames		I		Good frames

	total_frames		I		Total frames

	ua_hours			c*8		For numeric value, use GOOD_CHARGE

	user_name		c*(*)		User name

	user_day1		c*(*)		User telephone number during day – primary contact number

	user_day2		c*(*)		User telephone number during day – secondary contact number

	user_night		c*(*)		User telephone number during night

	user_inst		c*(*)		User institute

	tchan1		R*(ntc1+1)	Time channel boundaries (s); NTC1 = no. time channels

	dtchan1		R*(ntc1)	Widths of time channel boundaries (s); NTC1 = no. time channels

Format section

ver1		I		Format section version number

hdr		c*80		Header string (containing, in order: rid, user_name, short_title,

start_date, start_time, ua_hours)

add

form		I		Data format version number

Run section:

ver2		I		Run section version number

	run		I		Run number

	titl		c*80		Title

user		c*(*)(5)	Character cell array of length 5; the indivifual elements are

available using user_name, user_day1 … user_inst above

rpb		R(64)		Run parameter block (for integer elements of this mixed type field)

irpb		I(64)		Run parameter block (for integer elements of this mixed type field)

rrpb		R(64)		Run parameter block (for real elements of this mixed type field)

crpb		c*(*)(64)	Run parameter block (cell array for character elements of this

mixed type field)

Instrument section:

ver3		I		Instrument section version number

	name		c*(8)		Instrument name

ivpb		I(64)		Instrument variable parameter block (for integer elements)

rvpb		R(64)		Instrument variable parameter block (for real elements)

	ndet		I		No. detectors

	nmon		I		No. monitors

	nuse		I		No. user tables in DETECTOR.DAT

	mdet		I(nmon)	Detector indices of the monitors (Note, this is the internal index

number for the each detector i.e. the first column of wiring.dat)

monp		I(nmon)	Pre-scale values for monitors (8th column or wiring.dat)

	spec		I(ndet)		List of spectrum numbers for each detector (as spectra.dat)

	delt		R(ndet)	Delay time (as contained in col. 2 of detector.dat) *

	len2		R(ndet)	Secondary flight path (as in col. 3 of detector.dat) *

	code		I(ndet)		Code (as contained in col. 4 of detector.dat)

	tthe 		R(ndet)	Scattering angle (as contained in col. 5 of detector.dat) *

	ut1		R(ndet)	User table 1 (as contained in col. 6 of detector.dat) *

	ut2		R(ndet)	User table 2 (as contained in col. 7 of detector.dat) *

:

<for as many entries as the value of NUSE>

* Note that the order of the data is not necessarily the same as in the columns of detector.dat. The detector numbers corresponding to the elements of these arrays are given by the array udet in the Data acquisition electronics section. For aficionados: the order of the entries in udet is determined by the ‘detector index number’ given in the first column of wiring.dat. This is different from themore familiar detector number, which is contained in the second columnof wiring.dat.

Sample environment section

ver4		I		Sample environment section version number

spb		I(64)		Sample environment parameters (integer elements)

ispb		I(64)		Sample environment parameters (integer elements); ISPB  SPB

rspb		R(64)		Sample environment parameters (real elements)

cspb		c*(*)(64)	Sample environment parameters (cell array of character elements)

nsep		I		No. sample environment parameter blocks

------------------------------------------------------------------------------------------------------------------------

se01		I(32)		1st sample environment parameter block, integer elements

rse01		R(32)							 real elements

cse01		c*(*)(32)						 character elements

------------------------------------------------------------------------------------------------------------------------

se02

rse02

cse02

------------------------------------------------------------------------------------------------------------------------

:

<for as many entries as the value of NSEP>

Data acquisition electronics section

ver5		I		DAE section version number

daep		I(64)		Dae parameter block

	crat		I(ndet)		Crate numbers of the detectors (4th column of wiring.dat) *

	modn		I(ndet)		Module numbers of the detectors (5th column of wiring.dat) *

	mpos		I(ndet)		Positions in module of the detectors (6th col. of wiring.dat) *

timr		I(ndet)		Time channel regime numbers of the detectors (always =1; 3rd col.)

	udet		I(ndet)		Detector number of each detector (2nd column of wiring.dat) *

* See the comment in the Instrument section about the order of the elements in these arrays.

Time channel boundaries section

ver6		I		Time channel boundary section version number

ntrg		I		No. time channel bouindary regimes (always=1)

nfpp		I		??

	nper		I		No. periods

pmap		I		??

	nsp1		I		No. spectra

	ntc1		I		No. time channels

tcm1		I(5)		Time channel mode (0,1,2) – see footnote

tcp1		R(4,5)		Time channel parameters – see footnote

pre1		I		Clock prescale – see footnote

tcb1		I		Time channel boundaries in 1/32 s

Raw data section

ver8		I 		Raw data format version

	cnt1 		I		Counts

To return the entire array of counts, use:	>> c=gget(‘cnt1’)

For a section of the count array, use :	>> c=gget(‘cnt1[is_lo:is_hi]’)

See ‘Reading neutron counts’ in the manual for details about how counts are stored.

Notes on time channel boundaries:

 TCM1(1) = 0 :	time channel boundaries read from a file

 TCM1(1) =1 or 2 :	time channel boundaries generated from time channel parameters TCP1:

Up to 5 regimes of constant &/or logarithmic time channels:

TCM(i) = 1	constant width time channels generated between TCP1(1,i) and TCP1(3,i) by:

TCB1(n) = 32 × ( TCP1(1,i) + (n-1) × TCP1(2,i) – 4×DAEP(24) )  / PRE1

TCM(i) = 2	logarithmic width time channel generated between TCP1(1,i) and TCP1(3,i) by:

temp(1) = TCP1(1)

temp(n) = temp(n-1) × (1 + TCP1(2))  (n>1)

TCB1(n) = 32 × ( temp(n) – 4×DAEP(24) )  / PRE1

To convert TCB1 to s, use the algorithm:

TCHAN1(i) = (PRE1×TCB1(i) + 128×DAEP(24)) / 32

In MGENIE the time channel boundaries in s can be recovered directly using gget(‘tchan1’).

Appendix B: Troubleshooting

Some or all of the MGENIE commands fail to work

- Check that the MGENIE paths have been entered in startup.m as described in the introduction.

- If still not working, then remember to initialise MGENIE by typing >> mgenie

- Check that the command you are typing has not been used as the name of a variable. Some of the commands are very short, so this is easily done. To clear the variable my_var, for example, , type

>> clear my_var

The Graphics window is corrupted

This can sometimes happen if using MSLICE in the same Matlab session.

- Close the MGENIE graphics window by clicking on the ‘X’ in the top-right corner

- Reinitialise MGENIE, by typing >> mgenie

- Replot (your data will be unaffected)

.

Appendix C: MAPS alignment utilities

C.1 Utility functions

The following utilities can be used to analyse MAPS alignment data. Data are normalised by the monitor integral determined by a call to the  mgenie function analysis_init. Normally this call will be placed in the initialisation file (see ‘Introduction and Installation’):

To set the monitor integration parameters use:

>> analysis_init (mon_norm, mon_tlo, mon_thi, mon_norm_constant)

mon_norm                     	Default monitor number for normalisation

mon_tlo                     		Lower time integration limit (microseconds)

mon_thi                     		Upper time integration limit (microseconds)

mon_norm_constant           	Normalisation constant (i.e. the data will be normalised by

monitor integral in units of mon_norm_constant)

To view current settings:

>> analysis_init

DSPACE_MAPS

Creates an array of workspaces containing intensity as a function of d-spacing for the horizontal detector array of MAPS.

Data is normalised by the monitor integral. The monitor and integration ranges are set using the mgenie function analysis_init.

Assumes that the MID-TUBES spectrum mapping file has been used to maps detectors into spectra in the raw file.

Syntax:

>> w = dspace_maps (irun)	% uses period 1 and default dspacing range and step

>> w = dspace_maps (irun, period)   % uses default dspacing range and step

>> w = dspace_maps (irun, period, dspace_lo, dspace_hi)  % default dspacing step

>> w = dspace_maps (irun, period, dspace_lo, dspace_step, dspace_hi)

dspace_step +ve: constant step binning

dspace_step -ve: logarithmic binning

(see rebin function for more details of rebinning syntax)

DSCAN_ANAL

Analysis of an alignment scan. Produces an array of spectra containing the intensity as a function of d-spacing for the chosen instrument spectrum as a function of the scan parameter. The data must be either in a consecutive series of runs, or a single multi-period run. The user is prompted for parameters that are not supplied.

Data is normalised by the monitor integral. The monitor and integration ranges are set using the mgenie function analysis_init.

Syntax:

>> wout = dscan_anal(irun_start, specno, ...

scan_parameter_name, start_value, step, npnts)

DPLOT

Creates a colour plot of MAPS diffraction data created by DSPACE_MAPS or DSCAN_ANAL functions above. [The difference between dplot and the more standard mgenie function mplot is that the y axis is labelled with the scattering angle or scan parameter value rather than the index of the workspace in the array created by dspace_maps.]

Syntax:

>> dplot (w)

DGET_SPEC

Given the output of DSPACE_MAPS together with a scattering angle, returns the spectrum number which lies closest to that scattering angle. Also works with the output of DSCAN_ANAL, in which case it returns the spectrum number.

Syntax:

>> specno = dget_spec (w_array, value)

DINTEGRATE

Performs an integral of an array of spectra created with the DSCAN_ANAL or DSPACE_MAPS functions. The x-axis of the resulting spectrum is set as the scan parameter / scattering angle.[The standard mgenie function integrate will plot with the x-axis as the workspace index, not scan parameter / scattering angle].

Syntax:

>> wout = dintegrate (w, dlo, dhi)

C.2 Use in crystal alignment

- Use DSPACE_MAPS followed by DPLOT to look at single settings of a crystal

- Use DINTEGRATE to get the intensity as a function of scattering angle

- Read off the scattering angle of the peak, and use DGET_SPEC to get the spectrum number with scattering angle nearest to that value

- Use DSCAN_ANAL followed by DPLOT to look at the result of a scan

- Use DINTEGRATE to get the scan parameter corresponding to the peak position.