### ICSG/P05/2000

### C L R C

### RUTHERFORD APPLETON LABORATORY

### ISIS COMPUTING STEERING GROUP

### Raw File Fields

F A Akeroyd, S I Campbell, K J Knowles, C M Moreton-Smith

24 January 2000

Parameter Names

---------------

SECTION  ITEM  NAME    TYPE            CONTENTS

1        1   HDR     C*80            HEADER BLOCK

1        2   VER1    I*4             FORMAT version number

1        3   ADD     I*4(10)         SECTION addresses

2        1   VER2    I*4             RUN section version #

2        2   RUN     I*4             RUN #

2        3   TITL    C*80            RUN title

2        4   USER    C*20(8)         USER information

2        5   RPB     I*4(32)         RUN parameter block

3        1   VER3    I*4             INSTRUMENT section version #

3        2   NAME    C*8             INSTRUMENT name

3        3   IVPB  R/I*4(64)         INSTRUMENT parameter block

3        4   NDET    I*4             No. of detectors

3        5   NMON    I*4             No. of monitors

3        6   NUSE    I*4             NO. of USER defined (UTn) tables  ***B

3        7   MDET    I*4(nmon)       spectrum no. for each monitor

3        8   MONP    I*4(nmon)       prescale value for each monitor

3        9   SPEC    I*4(ndet)       SPECTRUM # table

3       10   DELT    R*4(ndet)       HOLD OFF table                    ***B

3       11   LEN2    R*4(ndet)       L2         table

3       12   CODE    I*4(ndet)       CODE for UTn tables               ***B

3       13   TTHE    R*4(ndet)       2Theta     table (scattering ang)

3       15   UT1     R*4(ndet)       USER defined table 1              ***B

..cont. to  UTn

4        1   VER4    I*4             SE section version #

4        2   SPB     I*4(32)         SAMPLE parameter block

4        3   NSEP    I*4             number of controlled SEPs

4        4   SE01    I*4(24)         SE parameter block #1

..cont. to  SEnn

5        1   VER5    I*4             DAE section version #

5        2   DAEP    I*4(64)         DAE parameter block

5        3   CRAT    I*4(ndet)       crate no.for each detector

5        4   MODN    I*4(ndet)       module no. for each detector

5        5   MPOS    I*4(ndet)       position in module for each detector

5        6   TIMR    I*4(ndet)       TIME REGIME # table

5        7   UDET    I*4(ndet)       'USER detector #' for each detector

6        1   VER6    I*4             TCB secton version #

6        2   NTRG    I*4             # of time regimes (normally =1)

6        3   NFPP    I*4             No. of frames per period

6        4   NPER    I*4             No. of periods

6        5   PMAP    I*4             period # for each basic period

6        6   NSP1    I*4             No. of spectra for time reg.= 1(tr=1)

6        7   NTC1    I*4             No. of time chan.              (tr=1)

6        8   TCM1    I*4(5)          time channel mode (0,1,2, ..) (tr=1) *A

6        9   TCP1    R*4(20)         time channel parameters       (tr=1) *A

6       10   PRE1    I*4             prescale value for 32MHz clock (tr=1)

(6.6 - 6.10 repeated for each time regime : NSP2,NTC2..etc.)

6       11   TCB1    I*4(ntc1+1)     Time channel boundaries        (tr=1)

(6.11 repeated for each time regime : TCB2,TCB3..etc.)

7            USER DEFINED  (max length =400 R*4 wds.)

8        1   VER7    I*4             DATA version #

8        2   DAT1    I*4(ntc1,nsp1)  raw data for first time regime

..cont. for NTRG time regimes

Detailed Listing of Sections

----------------------------

1     FORMAT

ITEM    NAME            TYPE    CONTENTS					COMFOR		MGENIE

----------------------------------------------------------------------------------------------------------

P    1       HDR(1:8)        c*8     RUN identifier (eg. LAD12345 )		rid		rid

-  3 characters for INSTRUMENT,

-  5 characters for RUN NUMBER.)

U            HDR(9:28)       c*20    User name                      		user

U            HDR(29:52)      c*24    Experiment short title         		s_title		short_title

P            HDR(53:64)      c*12    start date                     		date_start	start_date

P            HDR(65:72)      c*8     start time                     		time_start	start_time

HDR(73:80)      c*8     run duration (µA.Hr)     			ua_hours		uahr_char

--------------------------------------------------------------

F    2       VER1            I*4     format version number (ver=2)   ***B		form_ver1	{ver1}

--------------------------------------------------------------

P    3       ADD(1)          I*4     start address of RUN  section		add_run		{add}

P            ADD(2)          I*4     start address of INST section		add_inst

P            ADD(3)          I*4     start address of SE   section		add_se

P            ADD(4)          I*4     start address of DAE  section		add_dae

P            ADD(5)          I*4     start address of TCB  section		add_tcb

P            ADD(6)          I*4     start address of USER section   ***B		add_user

P            ADD(7)          I*4     start address of DATA section   ***B		add_data

P            ADD(8)          I*4     start address of LOG  section   ***B		add_log

ADD(9)          I*4  	spare					ifspare

FORM		I*4	data format flag (0 or 1)       ***C	mode_raw		{form}

0 - all TC for each spectrum

1 - the same TC from every spectrum

--------------------------------------------------------------

2     RUN

ITEM    NAME            TYPE    CONTENTS

--------------------------------------------------------------

F    1       VER2            I*4     RUN section version number			form_ver2	{ver2}

--------------------------------------------------------------

P    2       RUN             I*4     run number (starting from 1) 		run_num		{run}

--------------------------------------------------------------

U    3       TITL            C*80    run title					run_title	{titl}

--------------------------------------------------------------

U    2       USER(1)         C*20    user name					user_name	user_name

U            USER(2)         C*20    user telephone no. 1(day)			user_day1	user_day1

U            USER(3)         C*20    user telephone no. 2(day)       		user_day2	user_day2

U            USER(4)         C*20    user telephone no. (night)			user_night	user_night

U            USER(5)         C*20    user institution				user_inst	user_inst

C*20(3) spare					c1spare

C2spare

C3spare

--------------------------------------------------------------

P    4       RPB(1)          I*4     actual run duration 			run_durat	run_duration

U            RPB(2)          I*4     scaler for RPB(1),(22) (1=sec,..)		run_scaler	run_scaler

U            RPB(3)          I*4     test interval of RPB(2) (sec)		run_test

U            RPB(4)          I*4     dump interval				dump_int

U            RPB(5)          I*4     scaler for RPB(4)				dump_scaler

U            RPB(6)          I*4     test interval of RPB(5)(sec)			dump_test

U            RPB(7)          I*4     2**k (SNS frequency(Hz)=50/2**k) 		sns_k

P            RPB(8)          R*4     good proton charge (uA.hr)			good_charge	good_charge

P            RPB(9)          R*4     total proton charge (uA.hr)			total_charge	total_charge

P            RPB(10)         I*4     number of 'good' frames         		good_frames	good_frames

P            RPB(11)         I*4     total number of frames			total_frames	total_frames

U            RPB(12)         I*4     required run duration(units=RPB(1))***B	req_run

P            RPB(13)         I*4     actual run duration (seconds)      ***B	act_run

P            RPB(14)         I*4     monitor sum 1				mon1_sum

P            RPB(15)         I*4     monitor sum 2				mon2_sum

RPB(16)         I*4     monitor sum 3				irspare1

P            RPB(17-19)      C*12    finish date (dd-mmm-yyyy_)      		end_date		end_date

P            RPB(20-21)      C*8     finish time (hh-mm-ss)          		end_time		end_time

RPB(22)         I*4     RAL Proposal Number			irspare2(1)

RPB(- 32)       spare						irspare2(2-11)

--------------------------------------------------------------

3     INSTRUMENT

ITEM    NAME            TYPE            CONTENTS

--------------------------------------------------------------

F    1       VER3            I*4     INSTRUMENT section version no.(=2) ***B

--------------------------------------------------------------

F    2       NAME            C*8     instrument name

--------------------------------------------------------------

3               INSTRUMENT VARIABLE PARAMETER BLOCK

U            IVPB(1)         R*4     frequency chopper 1     (Hz)

U            IVPB(2)         R*4     frequency chopper 2     (Hz)

U            IVPB(3)         R*4     frequency chopper 3     (Hz)

U            IVPB(4)         I*4     delay c1                (µs)

U            IVPB(5)         I*4     delay c2                (µs)

U            IVPB(6)         I*4     delay c3                (µs)

U            IVPB(7)         I*4     max error on delay c1   (µs)

U            IVPB(8)         I*4     max error on delay c2   (µs)

U            IVPB(9)         I*4     max error on delay c3   (µs)

U            IVPB(10)        I*4     apperture c1

U            IVPB(11)        I*4     apperture c2

U            IVPB(12)        I*4     apperture c3

U            IVPB(13)        I*4     status c1  (run,stopped,stop open)

U            IVPB(14)        I*4     status c2  (run,stopped,stop open)

U            IVPB(15)        I*4     status c3  (run,stopped,stop open)

U            IVPB(16)        I*4     main shutter (open=1)

U            IVPB(17)        I*4     thermal shutter( " )

U            IVPB(18)        R*4     beam apperture horizontal(mm)

U            IVPB(19)        R*4     beam apperture vertical(mm)

U            IVPB(20)        I*4     scattering posn.(eg 1or2 HRPD)

U            IVPB(21)        I*4     moderator type no.

U            IVPB(22)        I*4     detector tank vacuum (1=vacuum)

U            IVPB(23)        R*4     L1

IVPB(24)	  R*4	  Rotor Frequency		HET

IVPB(25)	  R*4	  Rotor Energy			HET

IVPB(26)	  R*4	  Rotor Phase			HET

IVPB(27)	  I*4	  Rotor Slit Package		HET

IVPB(28)	  I*4	  Slow Chopper			HET

IVPB(29)	  R*4	  LOQ X centre			LOQ

IVPB(30)	  R*4	  LOQ Y centre			LOQ

IVPB(31)	  I*4	  Beam stop			LOQ

IVPB(32)	  R*4	  Radius Beam Stop		LOQ

IVPB(33)  	  R*4	  Source to detector distance	LOQ

IVPB(34)	  R*4	  FOE angle			LOQ

IVPB(35)	  R*4	  Angle of Incidence		CRISP

IVPB( - 64)		  spare

--------------------------------------------------------------

U    4       NDET    I*4             no.  of detectors

--------------------------------------------------------------

U    5       NMON    I*4             no.  of monitors

--------------------------------------------------------------

U    6       NUSE    I*4             no. of UTn  tables                 ***B

--------------------------------------------------------------

U    7       MDET    I*4(nmon)       detector nos. of the monitors

--------------------------------------------------------------

U    8       MONP    I*4(nmon)       prescale values for the monitors

--------------------------------------------------------------

U    9       SPEC    I*4(ndet)       spectrum # table

--------------------------------------------------------------

U    10      DELT    R*4(ndet)       'HOLD OFF' in (µs)           	***B

--------------------------------------------------------------

U    11      LEN2    R*4(ndet)       L2  table (m) (upstrem monitors -ve.

--------------------------------------------------------------

U    12      CODE    I*4(ndet)       code to define use of UT values    ***B

--------------------------------------------------------------

U    13      TTHE    R*4(ndet)       2Theta table (scattering angle)

--------------------------------------------------------------

U    13      UT1     R*4(ndet)       User defined table                 ***B

--------------------------------------------------------------

.

U     .      UTn     R*4(ndet)       User defined table                 ***B

--------------------------------------------------------------

4     SAMPLE ENVIRONMENT

ITEM    NAME    TYPE            CONTENTS

--------------------------------------------------------------

F    1       VER4    I*4             SE section version #    (=2)

--------------------------------------------------------------

2                               SAMPLE PARAMETER BLOCK

U            SPB(1)  I*4             position of sample changer

U            SPB(2)  I*4             sample type     (1 = sample+can

2 = empty can

3 = vanadium

4 = absorber

5 = nothing

6 = sample, no can)

U            SPB(3)  I*4             sample geometry (1 = cylinder

2 = flat plate

3 = HRPD slab)

U            SPB(4)  R*4             sample thickness normal to sampl.(mm)

U            SPB(5)  R*4             sample height (mm)

U            SPB(6)  R*4             sample width (mm)

U            SPB(7)  R*4             omega sample angle (deg)

U            SPB(8)  R*4             psi sample angle (deg)

U            SPB(9)  R*4             phi sample angle (deg)

U            SPB(10) R*4             scat.geom.(1=trans.2=reflect.   ***B

U            SPB(11) R*4             sample sCOH (barns)             ***B

U            SPB(12) R*4             sample sINC (barns)             ***B

U            SPB(13) R*4             sample sABS (barns)             ***B

U            SPB(14) R*4             sample number density (atoms.A-3)

U            SPB(15) R*4             can wall thickness (mm)         ***B

U            SPB(16) R*4             can    sCOH (barns)             ***B

U            SPB(17) R*4             can    sINC (barns)             ***B

U            SPB(18) R*4             can    sABS (barns)             ***B

U            SPB(19) R*4             can   number density (atoms.A-3)

U            SPB(20) C*40            sample name or chemical formula

SPB( -64)               spare

--------------------------------------------------------------

F=0  3       NSEP    I*4             no. of SE parameters

--------------------------------------------------------------

4       SE01(32)                SE parameter block #1           ***B

C*4(2)          name

I*4             value

I*4             value exponent

C*4(2)          units of value

I*4             low trip

I*4             high trip

I*4             current value

I*4             status (in bounds ?)

I*4             controlled parameter    (true/false)

I*4             run control parameter   (true/false)

I*4             log parameter changes   (true/false)

R*4             stability value (units per sec)

R*4             monitor repeat period

I*4             CAMAC location N

I*4             CAMAC location A

I*4             CAMAC offset (added to value)

I*4             CAMAC register group (1 or 2)

I*4             Pre process routine number

I*4(12)         CAMAC values

--------------------------------------------------------------

3+nsep  SEnsep  I*4(32)         SE parameter block #nsep

5     DATA ACQUISTION ELECTRONICS

ITEM      NAME    TYPE            CONTENTS

--------------------------------------------------------------

F    1       VER5    I*4             DAE section version #   (=2)

---------------------------------------------------------------

2               DAE PARAMETER BLOCK

U            DAEP(1) I*4             Word length in bulk store memory

F            DAEP(2) I*4             Length of bulk store memory (bytes)**A

U            DAEP(3) I*4             PPP minimum value                  ***B

P            DAEP(4) I*4             good PPP total (high 32 bits)   ***B

P            DAEP(5) I*4             good PPP total (low  32 bits)   ***B

P            DAEP(6) I*4             raw  PPP total (high 32 bits)   ***B

P            DAEP(7) I*4             raw  PPP total (low  32 bits)   ***B

P            DAEP(8) I*4             good ext. neut tot (high 32bits)***B

P            DAEP(9) I*4             good ext. neut tot (low  32 bits)***B

P            DAEP(10) I*4            raw  ext. neut tot (high 32 bits)***B

P            DAEP(11) I*4            raw  ext. neut tot (low  32 bits)***B

P            DAEP(12) I*4            ext. neutron gate (t1) (µs)   ***B

P            DAEP(13) I*4            ext. neutron gate (t2) (µs)   ***B

U            DAEP(14) I*4            detector for MON 1  (12 bits)   ***B

U            DAEP(15) I*4            module   for MON 1  ( 4 bits)   ***B

U            DAEP(16) I*4            crate    for MON 1  ( 4 bits)   ***B

U            DAEP(17) I*4            mask     for MON 1  (c4:m4:d12) ***B

U            DAEP(18) I*4            detector for MON 2  (12 bits)   ***B

U            DAEP(19) I*4            module   for MON 2  ( 4 bits)   ***B

U            DAEP(20) I*4            crate    for MON 2  ( 4 bits)   ***B

U            DAEP(21) I*4            mask     for MON 2  (c4:m4:d12) ***B

P            DAEP(22) I*4            total GOOD EVENTS (high 32 bits)***B

P            DAEP(23) I*4            total GOOD EVENTS (low  32 bits)***B

P            DAEP(24) I*4            frame synch delay (4µs steps) ***B

U            DAEP(25) I*4            frm snch origin(0:none/1:ext/2:int)***B

U		 DAEP(26) I*4            Secondary Master Pulse (0:en,1:dis)

U		 DAEP(27-29) I*4         External vetoes 0,1,2 (0 dis,1 en)

DAEP( -64)              Spare

--------------------------------------------------------------

U    3       CRAT    I*4(ndet)       crate no.for each detector

--------------------------------------------------------------

U    4       MODN    I*4(ndet)       module no. for each detector

--------------------------------------------------------------

U    5       MPOS    I*4(ndet)       posn. in module for each detector

--------------------------------------------------------------

U    6       TIMR    I*4(ndet)       TIME REGIME # table

--------------------------------------------------------------

U    7       UDET    I*4(ndet)       'USER DETECTOR #' for each det ***B

--------------------------------------------------------------

6     TIME CHANNEL BOUNDARIES

ITEM    NAME    TYPE            CONTENTS

--------------------------------------------------------------

F    1       VER6    I*4             TCB secton version #

--------------------------------------------------------------

U/P  2       NTRG    I*4             # of time regimes (normally =1)

--------------------------------------------------------------

F=1  3       NFPP    I*4             # of frames per period

--------------------------------------------------------------

F=1  4       NPER    I*4             # of periods

--------------------------------------------------------------

F=1  5       PMAP    I*4(256)        period # for each basic period

--------------------------------------------------------------

repeat following block for each time regime

time regime =1

--------------------------------------------------------------

U/P  3       NSP1    I*4             No. of spectra (+1 for zeroth)

--------------------------------------------------------------

U/P  4       NTC1    I*4             No. of time channels ( " )

--------------------------------------------------------------

U    5       TCM1    I*4(5)          time chan. mode (see footnote)

--------------------------------------------------------------

U    6       TCP1    R*4(4,5)                time chan. parameters (µs)

--------------------------------------------------------------

P    7       PRE1    I*4             prescale value for 32MHz clock (<=15)

--------------------------------------------------------------

time regime =2

--------------------------------------------------------------

11      NSP2    I*4          )

--------------------------------------------------------------

12      NTC2    I*4          )

--------------------------------------------------------------

13      TCM2    I*4(5)       )  As for 6.3,6.4,6.5,6.6,

--------------------------------------------------------------

14      TCP2    R*4(20)      )  6.7,6.8 for regime=2

--------------------------------------------------------------

15      PRE2    I*4          )

--------------------------------------------------------------

16      TCB2    I*4(ntc2+1)  )

--------------------------------------------------------------

...cont for additional time regimes

U    8       TCB1    I*4(ntc1+1)     time chan. bound. for TR=1(clock pulses)

--------------------------------------------------------------

U    8       TCB2    I*4(ntc1+1)     time chan. bound. for TR=2(clock pulses)

--------------------------------------------------------------

...cont for additional time regimes

TIME CHANNEL MODES:

0       Boundaries set by a table held in file TCB.DAT

1       TCB1(n) = (TCP1(1) + (n-1)*TCP1(2)-DAEP(24)*4)*32/PRE1

(ie. Dt = c )

(ie. less frm.synch delay)

2       temp(1)  = TCP1(1)

TCB1(1)  = (TCP1(1)-DAEP(24)*4)*32/PRE1 (ie. Dt = c.t

temp(n+1)= temp(n)*(1 + TCP1(2))

TCB1(n+1)= (temp(n+1) - DAEP(24)*4)*32/PRE1

7     USER DEFINED    (suggested layout)

ITEM    NAME    TYPE            CONTENTS

--------------------------------------------------------------

1       VER7    I*4             DATA version #

--------------------------------------------------------------

2       ULEN    I*4             USER_LEN

--------------------------------------------------------------

3       DAT1    R*4(user_len)   USER data

--------------------------------------------------------------

8      RAW DATA

Two formats exist for the data section. The earlier Version1 is

simply an array of consecutive data channels in 32bit integers.

Version2 compresses each spectrum to reduce the file size and

also has a data section header to describe the exact format.

8a     RAW DATA Version 1

ITEM    NAME    TYPE            CONTENTS

--------------------------------------------------------------

F    1       VER8    I*4             DATA version # (=1)

--------------------------------------------------------------

P    2       DAT1    I*4		raw data for first time regime

(ntc1+1)*(nsp1+1) channels

If data format flag=0

arranged (ntc1+1,nsp1+1)

(nsp1 spectra each containing

ntc1 data points)

If data format flag=1

arranged (nsp1+1,ntc1+1)

(for each time channel all

spectra stored together)

--------------------------------------------------------------

3       DAT2    I*4	       	raw data for 2nd time regime

(nt2+1,nsp2+1) channels

--------------------------------------------------------------

...cont for additional time regimes..

---Section Continued Overleaf---

8b     RAW DATA Version 2                                        ***C

ITEM    NAME    TYPE            CONTENTS

--------------------------------------------------------------

F    1       VER8    I*4             DATA version #  (=2)

--------------------------------------------------------------

2             Data Section Header Block

DHDR(1) I*4             Compression Type

0 no compression

1 byte relative

DHDR(2) I*4             Reserved

DHDR(3) I*4             Offset to Spectrum Descriptor Array

DHDR(4) I*4             Equivalent version 1 filesize (blocks)

DHDR(5) R*4             Compression Ratio for data section

DHDR(6) R*4             Compression Ratio for whole file

DHDR(7-32)              Unused

--------------------------------------------------------------

3             Spectrum Descriptor Array (spect=0..nspec)

DDES(2*spect+1) I*4     Number of words in compressed spectrum

DDES(2*spect+2) I*4     Offset to compressed spectrum

.

.   nspec = total of all spectra in all time regimes

.                 in all periods

--------------------------------------------------------------

4       DATA(0)     		Compressed raw data for each

.                      spectrum from each time regime

.                      for each period

.      		(**see below for details)

.	                On expansion a spectrum contains

.                      channel numbers from 0 to ntc

DATA(nspec)

---------------------------------------------------------------

***The data section now allows the use of a compressed format for the

data. At the time of writing only one compressed format is used. This is

byte relative. Each integer value in the data is stored as a relative offset

to the previous data point. This offset is stored within a byte thus allowing

offsets of + or - 127 between consecutive values. Should the offset

be outside this range then -128 is inserted in the byte as a flag and the

absolute value of the data point is stored in the next 4bytes. As the reduction

of space used for a value is from a 32bit integer to one byte, the maximum

compression ratio is 4.

STATUS SECTION

The status section is used during the run to store temporary values. It is

not copied to the raw data file.

It generally is placed in the gap between the FORMAT and RUN sections. The

table below names the parameters stored in the section.

ITEM    NAME    TYPE            CONTENTS

--------------------------------------------------------------

1               I*4             Run state (-1=paused,0=setup,

1=running)

--------------------------------------------------------------

2		I*4(2)		Start time in binary

--------------------------------------------------------------

3		I*4(2)		Paused time in binary

--------------------------------------------------------------

4		I*4(2)		Run time up to last pause

in binary

--------------------------------------------------------------

5		I*4		Internal error code

1	-	OK

other odd	Warning

even		error

--------------------------------------------------------------

6		I*4		VAX status code

--------------------------------------------------------------

7		I*4(5)		Monitor Spectrum Number

Start time in µsecs

End   time in µsecs

Start boundary

End   boundary

--------------------------------------------------------------

8		I*4		Status display flags for CAMAC

--------------------------------------------------------------

9		I*4		CAMAC heartbeat

--------------------------------------------------------------

10		I*4		SE run OK

--------------------------------------------------------------

11		I*4		Simulate flag

0 for normal running

1 for simulation mode

(no reads/writes to DAE)

--------------------------------------------------------------

12		I*4		No message flag

--------------------------------------------------------------

13		I*4		Hold flag (do not return till

end of run)

--------------------------------------------------------------

14		I*4		End wanted?

--------------------------------------------------------------

15		I*4(2)		Good frame up to last pause

Raw  frame

---Section Continued Overleaf---

--------------------------------------------------------------

16		I*4()		Maximum values allowed for CRPT

MAX_CRPT

MAX_DET

MAX_NUT

MAX_SPEC

MAX_CHAN

MAX_MON

MAX_VAX

MAX_DAE

MAX_SEP

MAX_TRG

MAX_UDAT

MAX_PER

MAX_TOTSPEC

--------------------------------------------------------------

17		I*4		Current Period Number

--------------------------------------------------------------

18		I*4		Number of channels in period

--------------------------------------------------------------

19		I*4(16)		16 time channel offsets

(One for each crate)

--------------------------------------------------------------

20		I*4		Monitor Sum of counts

--------------------------------------------------------------

Not provided, as this is identical to first element of USER array in RUN section

Renamed to make contents more obvious. Identical to first 24 characters of TITL in RUN section

Renamed for consistency with END_DATE in RUN section. 11 characters only (also for consistency)

Renamaed for consistency with END_TIME in RUN section

Renamed to make character format explicit, compared to GOOD_CHARGE in RUN section

To increase clarity of name